[Item: …………..]

SG600 Motorized ADA Compliant Swing Gate

Electric motor driven swing gate for barrier-free access. Compact accommodation of drive and control system in the center column.

Function:

- Comfortable contact free usage

- Complete opening of swing door for a convenient passage

- Quick opening times (2,0 s – 5,5 s depending on wing size and material)

- Automatic closing after set time span

- 2-way operation in entry and exit direction

- Addition to other access control systems for barrier-free access or luggage and material transport

- Ideal for usage in escape routes

- Installation as single or double swing door with a total passage width of up to 2400 mm

- Integrated pulse counter with separate counting for both directions

- Error history (logs) for technical service

- Indoor and outdoor usage

- Random generator for checking persons and bags

Design:

- Design housing with flowing lines, slender silhouette and tender forms

- Floor illumination (option)

- No visible connection elements

- Color design in basalt (sides) and anthracite (frame)

- Optional: customer-specific Color

Drive system/safety & security:

- Certified (TÜV Süd) low energy drive system (for maximum personal safety) with max. 1.69J, static opening and closing force max. 67N 
(according to prEN17352:2019, Performance Level d)ÖNORM EN 17352:2019-03-01

- Integrated drive system (brushless DC motor) for at least 10 million cycles (MCBF), certified according to EN 60335-2-103EN 60335-2-103:2015

- Duty cycle S1 (continuous operation), 100% according to DIN EN 60034OVE EN IEC 60034-1:2021-04-15

- Secure position or free rotation in case of alarm signal

- Free rotation in both directions in the case of power outage

- Automatic return to home position upon power return or reset of alarm signal

- Gate transmits warning notification in case of forced entry

Electric/Software: Integrated control system with display for parameterization and set-up

- Functions and parameters can be activated and parameterized as well as switched via I/O connections: passage direction, hold-open time, passage direction indication, illumination options, behavior in case of alarm, notification in case of forced entry

- Connection for 100 – 240 VAC, 50/60 Hz

- Connections for all current card reader systems

- 8 digital inputs with free allocable functions

- 4 digital outputs and 6 output relays for illumination control and status readout (freely allocable)

- 3 connector slots for extension modules

- Connectable via Ethernet interface

Housing:

- Aluminium profile with polyurethane varnishing

- Resistance against scratches, shocks and wear

- Resistant against corrosion, UV light, detergents and disinfectants

- Swinging element made from glass, acrylic glass or metal u-bar

- Protection class: IP54

Dimensions:

- Housing dimensions: 1092 x 178 mm (H x D)

- Passage width: 1000 mm (Standard) / 1200 mm (wide lane) / Specials on request

- Barrier height: 994 mm (Standard) / 1800 mm (full-height version) / Specials on request

- Total unit dimension for standard passage width (1000 mm): 1118 x 178 x 1092 mm (W x D x H)

- Installation space for card reader in card reader post: 60 x 80 x 40 mm (W x H x D)

- Opening angle adjustable from 0° to 300°

Installation:

- Screw mounting on stable floor surface

- Alternative mounting on unfinished floor by use of adjustable foundation frame (see accessories)

- Alternative glue mounting by use of a metal base plate (see accessories)

- Paving stone mounting (see accessories)

- Platform (see accessories)

Temperature range: 		-30 to +55 °C

Weight:				approx. 89lbs

Installation execution by manufacturer trained installation specialist, including user training and hand-over of inspection manual, operation manual declaration of conformity according to EC machinery directive/ low voltage directive, EMC directive